Routine Calling

Marine Rescue Greenwell Point,  (x2-3)
this is Mariah VMW 2120 (x2-3) on channel 16 over

Mariah, Mariah, Mariah, VMW 2120
This is Marine Rescue Greenwell Point, Marine Rescue Greenwell Point, Marine Rescue Greenwell Point
Received, change to channel 72

Marine Rescue Greenwell Point this is Mariah on channel 72

Mariah, this is Marine Rescue Greenwell Point, go ahead

==================================================================

MAYDAY

MAYDAY MAYDAY MAYDAY

THIS IS SCAMP SCAMP SCAMP VMQ3932 X 3

MAYDAY

SCAMP VMQ3932

50 nautical miles due east from danger point

Sinking after hitting a submerged object, estimate further 15 minutes afloat,

20 metre motor cruiser, white hull, 4 POB, EPIRB activated    over

==================================================================

Acknowledgment of receipt of a distress message by a vessel or a limited coast radio station should be made in the following way:

MAYDAY

Scamp VMQ3932 (x3)

THIS IS

Mariah, Mariah, Mariah VMQ 2120

Received Mayday

We are 15 nm due east of you, our eta to your location approximately one hour

Over

==================================================================

MAYDAY RELAY

MAYDAY RELAY  MAYDAY RELAY  MAYDAY RELAY

This is Why Knot VMQ 4400, Why Knot VMQ4400, Why Knot VMQ 4400

MAYDAY SCAMP VMQ3932

50 nautical miles due east from danger point

Sinking after hitting a submerged object, estimate further 15 minutes afloat,

20 metre motor cruiser, white hull, 4 POB, EPIRB activated

over

==================================================================

This message is to announce resumption of normal radio working:

MAYDAY;

ALL STATIONS, all stations all stations

THIS IS;

Mariah VMW2120, Mariah VMW2120, Mariah VMW2120

At 1000 vessel SCAMP VMQ3932 issued a mayday
SEELONCE FEENEE

Over

==================================================================