# EXERCISE 16: POSITION – THREE BEARING FIX WITH SET AND DRIFT - ANSWERS

1) Convert Compass bearings and course to True. Remember we need to apply Variation and Deviation for the compass course, as you are using the ship’s compass. But only apply Variation for the handheld compass bearings, as we assume no Deviation for handheld compass bearings.

|  | Course | Hook Island | Double Cone Island | Mt Merkara |
| --- | --- | --- | --- | --- |
| C | 359°C | 057°C | 322°C | 174°C |
| D | 2°W | - | - | - |
| M | 357°M | 057°M | 322°M | 174°M |
| V | 8°E | 8°E | 8°E | 8°E |
| T | 005°T | 065°T | 330°T | 182°T |

- Fix Position (A) = 20° 10.4’S  148° 46.05’E

- 20° 10’S  148° 44.15’E

- CMG = 024°T

- SMG = 5.6 knots

- Set = 103°T

- Drift Rate: 1.8 knots

We can allow a couple of degrees either way for bearings, set, courses, due to the boat’s yawing. The Lat/Long answers, if you are close (within one tenth of a nautical mile) that is good as you have understood the technique, precision comes with practice. (One tenth of a minute is one-tenth of a nautical mile, which equals (1852 metres / 10) = 185.2 metres.