Emergency Radio Procedure

A Distress Call has priority over all other transmissions

The obligation to accept a distress call is absolute and must be accepted with priority over all other communications

A radiotelephony alarm signal, distress call and a distress message from a vessel may be transmitted only on the authority of the master or skipper or the person responsible for the safety of that vessel

A distress call and message follows an alert/alarm signal

A distress call has absolute priority over all other transmissions and indicates that the vessel or person using it is threatened by grave and imminent danger and requests immediate assistance

All stations which hear the distress call must immediately cease all transmissions capable of interfering with distress communications, and must to continue to listen on the frequency on which the distress was received

International frequencies for distress are:

VHF: channel 16

HF/MF: 2182, 4125, 6215, 8291, 12 290, 16 420 kHz in MF/HF bands

Australian frequencies: Channel 67 on VHF (supplementary to Channel 16)

27.88 MHz in 27 MHz band
27.86 MHz in 27 MHz band (supplementary to channel 88)

The main and supplementary VHF radiotelephone channels used for distress and safety in Australia are 16 and 67

The order of priority of marine communications traffic is

Distress, Urgency, Safety, Routine

Distress: MAYDAY

Urgency: PAN PAN

Safety: SECURITE

A craft or person is in grave and imminent danger

It does not extend to situations where immediate assistance is sought on behalf of a person, for example, in a medical emergency

FIRST: Say MAYDAY (up to x3)

SECOND: Provide position:  Latitude/Longitude or true bearing and distance, or a precise geographical location

THIRD: Nature of Distress, POB, EPIRB ACTIVATED

If afloat and drifting, the rate and direction of drift could be stated

A distress call and message should be addressed to all stations or an individual on the distress channel

The distress signal MAYDAY should be used to precede each call and message