Towing

At times you may be asked to tow a vessel. There are a few points to keep in mind:

- Assess the risk for undertaking an emergency tow.

- If there are better equipped vessels nearby ask them to take the tow.

- If there is no option attach tow line to ski pole mount or stern bollard mount using lighterman’s hitch. (Attachment point will depend on your vessel, it must be a reinforced strong point where possible).

- Direct people in towed vessel, where and how to attach towline, prior to throwing the line.

- If appropriate, take onboard any people that maybe in danger (from the water).

- Set up towing light/signals where appropriate.

- Inform people in towed vessel how you intend to pass the tow line to them, e.g. a floating line downstream.

- Ensure there is a method to slip the tow line if necessary and a sharp knife to cut the line in an emergency.

- Take up tow gently/gradually.

- Maintain catenary in line.

- One person to constantly watch the tow.

- Stand clear of line.

- Navigate vessel to nearest, suitable and safe wharf.

Collision Regulations – Towing

Night: A yellow light should be shown above the white stern light (same arc as stern light), plus 2 white masthead lights on the vessel carrying out the tow.
Day: a black diamond on both vessels if available

There are further lights/shapes for larger vessels and longer tow. You must carry a copy of the International Collision Regulations on board.